Detached house until 1964

{

"image_path": "1964_Detached House_Red fired clay brick.png",

"label": "This is a Detached House, a Single-family house, from the 1964 and the facade is made out of red fire clay brick."

},

{

"image_path": "1964_Detached House_White fired clay brick",

"label": "This is a Detached House, a Single-family house, from the 1964 and the facade is made out of white painted fire clay brick."

}

General Overview:

- Detached houses built up to and including 1964 account for 418,000 homes, about 5.6% of the total housing stock in the Netherlands.

- 88% are owner-occupied, 12% are rented.

- These homes typically have 4 to 6 rooms over 2 to 4 floors.

- Often have a wooden ground floor.

Original Energy Performance (At Construction):

- No energy efficiency standards existed before 1965.

- No insulation was applied.

- Homes built before 1930 usually had no cavity walls.

- Many homes (especially pre-1960) had steel or softwood frames with single glazing.

- Heating was typically via local gas stoves, electric boilers, and natural ventilation.

- Central heating was rare.

Current Energy Performance:

- Most homes have been retrofit with insulation to some extent.

- Nearly all now have central heating with individual systems:

  - 95% use a high-efficiency gas boiler (HR107),

  - 1% use an older HR100 boiler,

  - 2% use electric heat pumps.

- For hot water:

  - 94% use a gas combi boiler,

  - 2% electric boilers,

  - 2% heat pumps.

- Window insulation upgrades:

  - 49% double glazing, 29% HR++ glazing,

  - 13% still single-glazed.

- Wall/floor/roof insulation:

  - Façade: 58% insulated,

  - Floor: 47%,

  - Sloped roof: 70%,

  - Flat roof: 57%.

- Ventilation:

  - 90% still use natural ventilation,

  - The rest have mechanical exhaust systems.

  - Some homes have airtightness improvements.

Technical Details (Example House):

- Usable area: ~153 m²

- Building height: 9.00 m

- Shape factor: 2.24 (ratio of heat loss surface to usable floor area)

Energy Performance Packages (Retrofit Options):

| Package | Heating System | Insulation | Ventilation | Energy Need (kWh/m²) |
| --- | --- | --- | --- | --- |
| 1 | Gas boiler (HR107) | Good insulation | Natural + mechanical exhaust | 213.6 |
| 2 | Electric heat pump | Good insulation | Natural + mechanical exhaust | 79.5 |
| 3 | Electric heat pump | Best insulation | Full mechanical with heat recovery | 62.3 |

Energy Label Distribution (from 240 sample homes):

- Most homes are in label D, E, F, G categories.

- Very few achieve A-labels without significant upgrades.

- Label G is the most common, followed by E and F.

Detached house 1965 and 1974.

General Overview:

- Detached houses built between 1965 and 1974 make up 1.7% of the Dutch housing stock (~128,500 homes).

- These homes are owner-occupied.

- Typically feature 4 to 6 rooms across 2 to 4 floors.

Original Energy Performance (At Construction):

- Since 1965, there have been energy standards, but these homes were poorly insulated by today’s standards.

- Natural ventilation was used.

- Central heating became more common over time.

Current Energy Performance:

- Many have since been upgraded:

  - All now have central heating with individual systems.

  - Most (89%) use a high-efficiency gas boiler (HR107).

  - Others use CR-boilers (6%) or electric heat pumps (3%).

- Hot water systems:

  - Mainly gas combi boilers (85%),

  - Also electric boilers (4%), water heaters (9%), and heat pumps (2%).

- Insulation upgrades:

  - Glazing: 58% double glazing, 24% HR++ glazing.

  - Other insulation:

    - Façade: 73% insulated,

    - Floor: 35%,

    - Sloped roof: 76%,

    - Flat roof: 57%.

- Ventilation:

  - 84% natural ventilation,

  - 14% mechanical exhaust,

  - 3% balanced ventilation.

  - Some have airtightness improvements.

Technical Details (Example House):

- Usable area: ~175 m²

- Building height: 9.00 m

- Shape factor: 2.12 (higher = more heat loss)

Energy Performance Packages (Retrofit Options):

| Package | Heating System | Insulation | Ventilation | Energy Need |
| --- | --- | --- | --- | --- |
| 1 | Gas boiler (HR107) | Good insulation | Natural + mechanical exhaust | 204.9 kWh/m² |
| 2 | Electric heat pump | Good insulation | Natural + mechanical exhaust | 72.8 kWh/m² |
| 3 | Electric heat pump | Best insulation | Full mechanical with heat recovery | 56.0 kWh/m² |

Energy Labels Distribution (from 93 sample homes):

- Labels range from A++++ to G

- Most common: Label C, D, E

- Few homes achieve A-label levels without significant retrofitting

Detached house 1975 and 1991.

General Overview:

- 201,000 detached houses built from 1975 to 1991, making up 2.7% of the Dutch housing stock.

- 95% are owner-occupied, only 5% are rented.

- Typically 4 to 7 rooms, spread across 2 to 4 floors.
Over half of these homes have three floors.

- Mostly traditionally built, but with increasing variety in design and structure.

Original Energy Performance (At Construction):

- Building codes improved during this period:

  - 1975: First insulation standards (Rc 1.3 m²K/W for roofs/walls),

  - 1979: Double glazing required in living areas,

  - 1983: Floor insulation introduced,

  - 1988: Higher insulation for roofs/walls (Rc 2.0).

- However, many homes from the earlier part of this period were poorly insulated by modern standards.

- Ventilation was natural (open windows, grilles),
and central heating became more common over time.

Current Energy Performance:

- All homes now have central heating with individual systems:

  - 86% HR107 boilers,

  - 6% HR100 boilers,

  - 3% use electric local heating,

  - 2% have heat pumps.

- Hot water systems:

  - 91% use combi boilers,

  - 4% electric boilers,

  - 5% gas water heaters (geysers).

- Glazing:

  - 66% of windows have double glazing,

  - 20% have HR++ glass,

  - Only 4% is still single-glazed (compared to 13% in pre-1965 homes).

- Insulation:

  - All solid walls are insulated, though not always to high standards.

  - Floor insulation: 80%,

  - Sloped roof: 89%,

  - Flat roof: 55%.

- Ventilation:

  - 78% natural,

  - 21% mechanical exhaust,

  - Homes include air tightness measures.

Technical Details (Example House):

- Usable floor area: 161 m²

- Building height: 9.00 m

- Shape factor: 2.18 (ratio of heat-loss surface to floor area)

Energy Performance Packages (Renovation Options):

| Package | Heating System | Insulation | Ventilation | Energy Need (kWh/m²) |
| --- | --- | --- | --- | --- |
| Current | HR107 gas boiler | Basic/moderate | Natural | 157.3 |
| Package 1 | HR107 gas boiler | Upgraded | Natural + mechanical exhaust | 75.3 |
| Package 2 | Electric heat pump | Upgraded | Natural + mechanical exhaust | 75.3 |
| Package 3 | Electric heat pump | Best insulation | Fully mechanical with heat recovery | 58.5 |

Energy Label Distribution (from 173 homes – WoON2018 survey):

- Majority are rated Label D (44.5%), with many also in Label C (12.9%) and Label E (15.2%).

- Very few homes achieve high labels (A++, A+, A), unless renovated.

- Only 0.6% reach A++++, 0.1% A++, and 1.2% A+.